| Station GUMA |  |  |  |
| --- | --- | --- | --- |
| Receiver | TOPCON GB1000 | INSU44 |  |
| Antenna |  | Antenna height | 1.210 |
| Owner | Géosciences Rennes | O. Dauteuil / F. Moreau |  |
| Location | In Guma lodge |  |  |
| Coordinates | Latitude | Longitude | Elevation |
|  | -18°57'43,993 | 22°22'21.922 | 991,01300 |
| Survey acquisition : | Record sampling | mask | Date |
|  | 30 sec | 5 deg | 12/10/2010 – 19h00 13/10/2010 – 8h45 |
| Site location: | In an open place, 150 m before the réception entrance, close to a sandy road and the house Inside the forest. |  |  |
| Benchmark: | Iron corner stacked into the ground, cross on the side of the iron. corner |  |  |