| Station MOMB |  |  |  |
| --- | --- | --- | --- |
| Receiver | TOPCON GB1000 | INSU45 |  |
| Antenna |  | Antenna height | 1.396 |
| Owner | Géosciences Rennes | O. Dauteuil / F. Moreau |  |
| Location | Primary school of Nokaneng |  |  |
| Coordinates | Latitude | Longitude | Elevation |
|  | -19°12'44.19" | 22°47'28.927 | 974,58502 |
| Survey acquisition : | Record sampling | mask | Date |
|  | 30 sec | 5 deg | 07/10/2010 – 12h30 14/10/2010 – 10h00 |
| Site location: | In an open, 200 m to the runaway |  |  |
| Benchmark: | Iron corner stacked into the ground, cross |  |  |